Experiment No. 5

ARDUINO BASED MANUAL ELECTRONIC COUNTER USING

PROTEUS

Objectives

- To understand the interfacing of an Arduino Uno with a seven-segment display.

- To implement a manual electronic counter using a push button.

- To simulate the counter circuit using Proteus software.

- To verify the Arduino Embedded C program for counting from 0 to 9.

PROGRAM:

int x0,x1,x2,x3,x4,x5,x6,x7,x8,x9; int delay_time=200;   void setup() {

//configure pin2 as an input and enable the internal pull-up resistor

pinMode(12, INPUT_PULLUP); pinMode(1,OUTPUT);

pinMode(2,OUTPUT); pinMode(3,OUTPUT); pinMode(4,OUTPUT); pinMode(5,OUTPUT); pinMode(6,OUTPUT); pinMode(7,OUTPUT); } void loop() {

//read the pushbutton value into a variable int sensorVal = digitalRead(12);

if (sensorVal == LOW) { x0=true; } while(x0){ zero(); sensorVal = digitalRead(12); if (sensorVal == LOW) { x1=true; x0=false; } } while(x1){ one(); sensorVal = digitalRead(12); if (sensorVal == LOW) { x2=true; x1=false;

}

} while(x2){ two(); sensorVal = digitalRead(12); if (sensorVal == LOW) { x3=true; x2=false; } } while(x3){ three(); sensorVal = digitalRead(12); if (sensorVal == LOW) { x4=true; x3=false; } } while(x4){ four(); sensorVal = digitalRead(12); if (sensorVal == LOW) { x5=true; x4=false; } } while(x5){  five(); sensorVal = digitalRead(12); if (sensorVal == LOW) { x6=true;

x5=false; } } while(x6){  six(); sensorVal = digitalRead(12); if (sensorVal == LOW) { x7=true; x6=false; } } while(x7){ seven(); sensorVal = digitalRead(12); if (sensorVal == LOW) { x8=true; x7=false; } } while(x8){ eight(); sensorVal = digitalRead(12); if (sensorVal == LOW) { x9=true; x8=false; } } while(x9){ nine(); sensorVal = digitalRead(12);

if (sensorVal == LOW) { x0=true; x9=false; }

}

}

void zero() { for(int i=1;i<7;i++) { digitalWrite(i,HIGH); digitalWrite(7,LOW); } delay(delay_time);

}

void one() { digitalWrite(1,LOW); digitalWrite(2,HIGH); digitalWrite(3,HIGH); digitalWrite(4,LOW); digitalWrite(5,LOW); digitalWrite(6,LOW); digitalWrite(7,LOW); delay(delay_time);

}

void two() { digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,LOW); digitalWrite(4,HIGH); digitalWrite(5,HIGH); digitalWrite(6,LOW); digitalWrite(7,HIGH); delay(delay_time);

}  void three() { digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,HIGH); digitalWrite(4,HIGH); digitalWrite(5,LOW); digitalWrite(6,LOW); digitalWrite(7,HIGH); delay(delay_time);

}

void four() { digitalWrite(1,LOW); digitalWrite(2,HIGH); digitalWrite(3,HIGH); digitalWrite(4,LOW);  digitalWrite(5,LOW); digitalWrite(6,HIGH); digitalWrite(7,HIGH); delay(delay_time);

}

void five() { digitalWrite(1,HIGH); digitalWrite(2,LOW); digitalWrite(3,HIGH); digitalWrite(4,HIGH); digitalWrite(5,LOW); digitalWrite(6,HIGH); digitalWrite(7,HIGH); delay(delay_time);

}

void six() { digitalWrite(1,HIGH); digitalWrite(2,LOW); digitalWrite(3,HIGH); digitalWrite(4,HIGH); digitalWrite(5,HIGH); digitalWrite(6,HIGH); digitalWrite(7,HIGH); delay(delay_time);

}

void seven() { digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,HIGH); digitalWrite(4,LOW); digitalWrite(5,LOW); digitalWrite(6,LOW); digitalWrite(7,LOW); delay(delay_time);

}

void eight() { digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,HIGH); digitalWrite(4,HIGH); digitalWrite(5,HIGH); digitalWrite(6,HIGH); digitalWrite(7,HIGH); delay(delay_time);

}

void nine() { digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,HIGH); digitalWrite(4,HIGH); digitalWrite(5,LOW); digitalWrite(6,HIGH); digitalWrite(7,HIGH); delay(delay_time);

}

OUTPUT: